# TAPI Server Test

## Abstract

The TAPI server test, tapiserver.dll, verifies that the peripheral supports Telephony API (TAPI) functionality, especially the ability to complete inbound calls.

TapiServer is a functional/stress test that will initialize TAPI and monitor one or more lines for incoming calls.  If a call is offered, it will be connected using the “lineAnswer” API and then dropped.

## Hardware Requirements

| Requirements | Description |
| --- | --- |
| Two TAPI devices | One for the server, which the test will be runing, and one for making the call. You may use an External modem, PCMCIA modem, null modem, or IR modem. |

## Required Software

| tapiserver.dll | Test dll file. |
| --- | --- |
| A TAPI client Application | You may use tapiclient for making the calls. |

## Running the Test

The following command line shows the syntax for running the test.

tux –o –d tapiserver –c “[parameters]” –x TestNumber

example: tux -o -d tapiserver -c "-d0 –x3" -x 100

The following table shows the general test case parameters.

| Parameter | Description |
| --- | --- |
| -d device_identifier | Sets the TAPI device identifier of the line to be tested. A list of device identifiers may be obtained by using the –l parameter. The default value is 0, which is the external modem on most standard Windows CE configurations. |
| -x minutes | Quits after x minutes. Default: loop forever |
| -t seconds | Drop a call after t seconds. Default: hold untill caller drops |
| -m media mode | Sets the media mode 0: Data modem (default) 1: Interactive voice 2 = Automated voice 3 = Group 3 FAX 4 = Group 4 FAX 5 = Digital data 6 = TDD 7 = Teletex 8 = Videotex 9 = Telex 10 = ISDN mixed 11 = ADSI 12 = VoiceView 13 = Unclassified |
| -h | Prints a short help description. |

The following are additional test features:

## Test Cases

| Test Case ID | 100: Run the Default Test |
| --- | --- |
| Test Description | This test will open the device specified in the command line. It will listen for incoming connections, answer the connection and then drop it (if the –t parameter is specified). |
| Troubleshooting | This test will fail if the application, TAPI, or service-provider versions are incompatible. |