WaitForPrinterChange Specification

AlbertT 2.1

Background

The WaitForPrinterChange() API is currently implemented as a synchronous RPC call.  The RPC call blocks in the server until there is a printer change or a time-out, causing an large, unacceptable number of threads in the spooler process.  Furthermore, one thread is also required for each printer change notification needed in the client process.  In the current implementation:

A printer server with 25 printers and printman viewing all 25 servers uses:

- 27 printman threads (1 UI, 1 local printer changes, 25 for printers)

- >25 spooler threads, one for each printer

For each window that is opened against a printer on this server, an additional thread will be created on the server machine.

Overview

The client calls the router, which creates an event in the client address space and one locally, then calls the print provider.  Instead of using synchronous RPC to register and return on notification, the print provider call returns immediately.  On registration, the print server sets up a context handle with the client (now the server is acting like a client) then returns immediately.  On notification, the provider calls the router directly with the notification using this context handle.  The router sets the event and increments the notification count.

When the client calls asks for the next notification, the router decrements the notification count and sets the event appropriately.  The current implemention will group multiple notifications together: if three notification occur before the user issues the FindNext, the three events will be handled as one.

Interface

One of the goals of the API specification is to closely follow the existing base interfaces.  The FindPrinterChangeNotification() is modeled after FindFirstChangeNotification base APIs.

FindFirstPrinterChangeNotification (Win32)

HANDLE FindFirstPrinterChangeNotification(hPrinter, fdwFilter, fdwFlags)

HANDLE hPrinter;	/* handle of printer to watch	*/

DWORD fdwFilter;	/* filter conditions to watch for	*/

DWORD fdwFlags;           /* flags */

The FindFirstPrinterChangeNotification function creates a change notification handle and sets up initial change notification filter conditions. A wait on a notification handle succeeds when a change matching the filter conditions occurs in the specified printer.

Parameter	Description

hPrinter		Handle to a printer the user wishes to watch.

fdwFilter	Specifies the filter conditions that satisfy a change notification wait. This parameter

can be one or more of the following values:

Value	Meaning

PRINTER_CHANGE_PRINTER	Notify the application of any changes to a printer.

PRINTER_CHANGE_JOB	Notify the application of any changes to a job.

PRINTER_CHANGE_FORM	Notify the application of any changes to a form.

PRINTER_CHANGE_PORT	Notify the application of any changes to a port.

PRINTER_CHANGE_PRINT_PROCESSOR	Notify the application of any changes to a print processor.

PRINTER_CHANGE_PRINTER_DRIVER	Notify the application of any changes to a printer driver.

fdwFlags	This parameter should be zero.

Returns

If the function succeeds, the return value is a handle of a find change notification object; otherwise, it is INVALID_HANDLE_VALUE. To get extended error information, use the GetLastError function.

Comments

The WaitForSingleObject and WaitForMultipleObjects functions monitor the specified printer by using the handle returned by the FindFirstPrinterChangeNotification function. A wait is satisfied when one of the filter conditions occurs in the monitored printer.

After the wait has been satisfied, the application can respond to this condition and continue monitoring the printer by calling the FindNextPrinterChangeNotification function and the appropriate wait function. When the handle is no longer needed, close it by using the FindClosePrinterChangeNotification function.

See Also

FindClosePrinterChangeNotification, FindNextPrinterChangeNotification, WaitForMultipleObjects,

WaitForSingleObject

FindNextPrinterChangeNotification (Win32)

BOOL FindNextPrinterChangeNotification(hEvent, dwReserved, lpvReserved, lpdwChange)

HANDLE hEvent;	/* handle of change notification to signal	*/

LPDWORD lpdwChange;  /* indicates change on printer */

DWORD dwReserved;      /* reserved */

LPVOID lpvReserved;      /* reserved */

The FindNextPrinterChangeNotification function requests that the operating system signal a change

notification handle the next time it detects an appropriate change.  It also returns the status of the previous notification.

Parameter	Description

hEvent	Specifies a change notification handle created by the FindFirstPrinterChangeNotification function.

dwReserved	Reserved; must be zero.

lpvReserved	Reserved; must be NULL.

lpdwChange	Points to a DWORD that recieves the change result.  The result can be the composite of the following values:

PRINTER_CHANGE_ADD_PRINTER	A printer was added to the server.

PRINTER_CHANGE_SET_PRINTER	A printer was set.

PRINTER_CHANGE_DELETE_PRINTER	A printer was deleted.

PRINTER_CHANGE_ADD_JOB	A job was sent to a printer.

PRINTER_CHANGE_SET_JOB	A job was set.

PRINTER_CHANGE_DELETE_JOB	A job was deleted.

PRINTER_CHANGE_WRITE_JOB	Job data was written.

PRINTER_CHANGE_ADD_FORM	A form was added to the server.

PRINTER_CHANGE_SET_FORM	A form was set on the server.

PRINTER_CHANGE_DELETE_FORM	A form was deleted from the server.

PRINTER_CHANGE_ADD_PORT	A port or monitor was added to the server.

PRINTER_CHANGE_CONFIGURE_PORT	A port was configured on the server.

PRINTER_CHANGE_DELETE_PORT	A port or monitor was deleted from the server.

PRINTER_CHANGE_ADD_PRINT_PROCESSOR	A print processor was added to the server.

PRINTER_CHANGE_DELETE_PRINT_PROCESSOR	A print processor was deleted from the server.

PRINTER_CHANGE_ADD_PRINTER_DRIVER	A printer driver was added to the server.

PRINTER_CHANGE_DELETE_PRINTER_DRIVER	A printer driver was deleted from the server.

PRINTER_CHANGE_TIMEOUT	A timeout occurred.

Returns

If the function succeeds, the return value is TRUE; otherwise, it is FALSE. To get extended error

information, use the GetLastError function.

Comments

The result of the change is returned in lpdwChange.  After the FindNextPrinterChangeNotification function returns successfully, the application can wait for notification that a change has occurred by using the WaitForSingleObject or WaitForMultipleObjects function.

If a change occurs after a call to FindFirstPrinterChangeNotification but before a call to FindNextPrinterChangeNotification, the operating system records the change. When FindNextPrinterChangeNotification is executed, the recorded change immediately satisfies a wait for the

change notification.

FindNextPrinterChangeNotification should be called after the event returned from FindFirstPrinterChangeNotification has been set.  This api will reset the event, and it will be set when another change occurs.  When hEvent is no longer needed, close it by using the FindClosePrinterChangeNotification function.

See Also

FindClosePrinterChangeNotification, FindFirstPrinterChangeNotification, WaitForMultipleObjects,

WaitForSingleObject

FindClosePrinterChangeNotification (Win32)

BOOL FindClosePrinterChangeNotification(hEvent)

HANDLE hEvent;	/* handle of change notification to close	*/

The FindClosePrinterChangeNotification function stops notification handle change monitoring.

Parameter	Description

hEvent	Identifies a change notification handle created by the FindFirstPrinterChangeNotification function.

Returns

If the function succeeds, the return value is TRUE; otherwise, it is FALSE. To get extended error

information, use the GetLastError function.

Comments

After the FindClosePrinterChangeNotification function is called, the handle specified by the hEvent

parameter cannot be used in subsequent calls to either the FindNextPrinterChangeNotification or

FindClosePrinterChangeNotification function.

See Also

FindFirstPrinterChangeNotification, FindNextPrinterChangeNotification, WaitForMultipleObjects,

WaitForSingleObject

Deltas per component

| Winspool.drv |  |
| --- | --- |
| Delta | Maps hEvent from FFPCN in client address space to hPrinter so that RPC communication can be established. |
| Entrypoints | FindFirstPrinterChangeNotification hPrinter fdwFilter fdwOptions FindNextPrinterChangeNotification hEvent dwReserved lpvReserved lpdwChange FindClosePrinterChangeNotification hEvent |
| Callouts | RpcFindFirstPrinterChangeNotification RpcFindNextPrinterChangeNotification RpcFindClosePrinterChangeNotification |
| State | hEvent hPrinter pairs |

| Server/router |  |
| --- | --- |
| Delta | Calls provider with notification request. If local, creates event pair. On notification, sets event and updates counter. On FNPCN, decrements the counter and resets the event if necessary. On FCPCN calls the provider to shut down the notification. |
| Entrypoints | Called from Client (winspool.drv) ClientFindFirstPrinterChangeNotification hPrinter fdwFilter fdwFlags dwPID pdwEvent FindNextPrinterChangeNotification hPrinter lpdwChange dwReserved lpvReserved FindClosePrinterChangeNotification hPrinter ReplyPrinterChangeNotification hNotify fdwFlags cbBuffer pBuffer Providor to Router registration ProvidorFindFirstPrinterChangeNotification hPrinter fdwFlags fdwOptions hChange ProvidorFindClosePrinterChangeNotification hPrinter Router to Router communication RouterFindFirstPrinterChangeNotification hPrinter fdwFlags pszLocalMachineName dwSpool (hPrinterRemote) RouterReplyPrinterChangeNotification hNotify fdwFlags cbBuffer pBuffer Calls from Print servers to clients for notifications ReplyOpenPrinterChangeNotification [RPC interface takes a machine name also] hPrinter phNotify ReplyClosePrinterChangeNotification hNotify |
| Callouts | Provider->FindFirstPrinterChangeNotification Provider->FindClosePrinterChangeNotification |
| State | pSpool: hEvent, dwCount (NULL hEvent = remote) |

| Provider |  |
| --- | --- |
| Delta | Calls ReplyPrinterChangeNotification |
| Entrypoints | FindFirstPrinterChangeNotification hPrinter fdwFlags pszLocalMachine hNotify FindClosePrinterChangeNotification hPrinter |
| Callouts | ReplyPrinterChangeNotification |
| State |  |

Sequence of events

Registration

Client code

calls router (hPrinter, fdwFlags, pszLocalMachine, PID) [RPC Local]

if SUCCESS

create hEvent-hPrinter pair (hEvent returned from router)

else

return FAIL

Router

calls print provider (hPrinter, fdwFlags, pszLocalMachine, pSpool) [Function call]

if SUCCESS

create event with security based on impersonating thread

duplicate event into client address space

reset counter

return duplicated event

else

return FAIL

Provider

registers event

Remote case using win32spl:

Provider (win32spl)

calls router: CallRouterFindFirstPrinterChangeNotification

Router [local]

calls remote router: RouterFindFirstPrinterChangeNotification

Router [remote]

remembers machine name, options, and original printer handle

calls print providor

if ENDPOINT (fdwStatus & NOTIFY_STATUS_ENDPOINT)

setup context handle with client

return

Notification

Provider

calls router (hPrinter, fdwFlags, pszLocalMachine, pSpool) [Function call]

Router

if pszLocalMachine != SELF

call client router [RPC call]

else

increment notification count

set event

FindNextPrinterChangeNotification

Client

calls router (hPrinter) [RPC call]

Router

decrements notification count (zeros it if GROUPING set)

resets event if necessary

returns change notifications

FindClosePrinterChangeNotification

Client

destroys client event

calls router (hPrinter) [RPC call]

Router

if local

destroys router event

if hNotifyRemote exists

calls client router to close notify handle [RPC call]

calls print provider [Function call]

Provider

quits notification

Connectivity

Scenarios:

511 app

|  | Server |  |
| --- | --- | --- |
| Client | 511 | Daytona |
| 511 | Normal | Uses regular WaitForPrinterChange api. The RPC message goes to the remote router, which requests a FFPCN from the appropriate provider. |
| Daytona | Client translates call and attempts a FFPCN with 511, which fails. Call unwinds to winspool.drv which does the normal WPC to server. | Winspool.drv converts from WPC to FFPCN to minimize the number of threads created. Client thread waits on the FFPCN event. |

Daytona app

|  | Server |  |
| --- | --- | --- |
| Client | 511 | Daytona |
| 511 | Crash | Crash |
| Daytona | Client attempts a FFPCN with 511, which fails. Call unwinds to winspool.drv which creates an event and a thread which calls WPC and then sets the event and exits. | Normal |

Daytona server with 511 Providor

The router must deal with the 511 provider by failing back to winspool.drv and forcing it to create a thread and use WPC.  (Or use WPC directly if its a 511 app.)

Additional APIs

Open issues

Security

The client spooler should authenticate all notifications to ensure that the notification really did come from the server.  This is difficult to do, so instead, the client will check that the server has appropriate access to the notification.  When a notification is set up, the hEvent will be create with the client thread security descriptor.  The server spooler will keep the hToken of the client (from the impersonating thread) in the pSpool, then will impersonate before RPCing back to the client.  The client will do an access check against the event.  This bascially states that anyone with access to the client process will be able to send notifications to it.

Currently, delegation is not supported, so the notifications cannot be authenticated.  The client will usually query the server for change details, so this is not a major issue.

Since the notifications originate from the router running in local system, the spoolss pipe must now accept NULL sessions.

pPrintHandle needs to clean up pNotify and pChange, so it keeps pointers to them.  pChange and pNotify must notify pPrintHandle when they go away, so they keep pointers to pPrintHandle.

State:

A	No notifications

pPrintHandle->pChange 	= NULL

pPrintHandle->pNotify 	= NULL

pChange		= X

pChange->eStatus 	= X

pNotify			= X

pNotify->pPrintHandle	= X

B	Client Forming 1, Router Forming 1

pPrintHandle->pChange 	= V

pPrintHandle->pNotify 	= NULL

pChange		= V

pChange->eStatus 	= STATUS_FORMING

pNotify			= X

pNotify->pPrintHandle	= X

C	Client Forming 2

pPrintHandle->pChange 	= V

pPrintHandle->pNotify 	= V

pChange		= V

pChange->eStatus 	= STATUS_VALID | STATUS_CLIENT

pNotify			= V

pNotify->pPrintHandle	= V

C2	Client Forming 2 (ROPCN failed)

pPrintHandle->pChange 	= V

pPrintHandle->pNotify 	= NULL

pChange		= V

pChange->eStatus 	= STATUS_VALID | STATUS_CLIENT

pNotify			= X

pNotify->pPrintHandle	= X

D	Router Forming 2

pPrintHandle->pChange 	= V

pPrintHandle->pNotify 	= NULL

pChange		= V

pChange->eStatus 	= STATUS_VALID

pNotify			= X

pNotify->pPrintHandle	= X

E

Transitions:

AB	CFFPCN 1

RFFPCN 1

BC	CFFPCN 2 success

BC2	CFFPCN 2 success (ROPCN failed)		BAD STATE, eliminate

BA	CFFPCN 2 fail, RFFPCN 2 fail

BD	RFFPCN 2 success