# Windows NT Remote Access

# Modem/X.25/Switch DLL

# Component Design Document

Revision History

| Revision | Date | Author | Comment |
| --- | --- | --- | --- |
| 0.1 | May 18, 1992 | J. Perry Hannah | Original |
| 1.0 | June 04, 1992 | perryh | Incorporated design review comments |
| 1.1 | June 15, 1992 | perryh | New modem.inf format with modem responses section |
| 1.2 | June 18, 1992 | perryh | Append, carrierbaud and connectbaud macros, and continuation lines. New RasDeviceGetParams constraints. |
| 1.3 | July 15, 1992 | perryh | Attribute bits, macro tables, nested macros, error codes, RasDevResetCommand, RasDevCheckResponse. |
| 1.4 | July 24, 1992 | perryh | Variable attribute bit set by RasDevGetParams, macro delimiters encapsulated in Device INF APIs, device contol block, return codes and macros. |
| 1.5 | Aug 8, 1992 | perryh | w and match macros, RasDevCheckResponse dReceivedLen parameter |
| 1.6 | Aug 19, 1992 | perryh | Added Installation section |

Table of Contents

µ1.  Audience	1

1.1.  Prerequisites	1

1.2.  References	1

2.  The Problem	1

2.1.  Purpose	1

2.2.  Design Goals	1

2.3.  Design Exclusions	1

3.  Platform Support	2

3.1.  Systems	2

3.2.  Down-level Support	2

3.3.  Up-level Support	2

4.  Development Tools	2

4.1.  Build Environment	2

4.2.  Language Tools	2

4.3  RAS Device INF File Library	2

5.  The Solution	3

5.1.  Overview	3

5.2.  Major Data Structures	3

5.2.1  Device INF files	3

5.2.2  Attribute Bits	3

5.2.3  Data Structures	4

Device Control Block	5

Information Table	5

Macro Translation Table	5

5.3.  Pseudo-code and/or Block Diagrams	5

5.3.1  DeviceConnect() and DeviceWork()	5

5.3.2  DeviceEnum	8

5.3.3  DeviceGetInfo	9

5.4.  Modular Decomposition	9

6.  Testing	10

6.1.  Test Tools and Hooks	10

6.2.  Unit Testing	10

6.3.  Test Problems	10

7.  Dependencies	10

8. Intallation	10

9.  Work Estimates	11

10.  Open Issues	11

Appendix A  INF Files	12

1.  INF File Format	12

Definition	12

Example modem.inf	14

2.  Macros	15

Macros defined in Device INF API code	16

Macros defined in INF file and RASMXS.H	16

Return Codes and Macros	16

Appendix B   RAS Device INF File APIs	17

## 1.  Audience

### 1.1.  Prerequisites

The reader is assumed to be familiar with the Remote Access Service product in general and the RAS Manager in particular.

### 1.2.  References

NT RAS Manager Device DLLs Functional Specification describes the device DLL interface with RAS Manager.

NT RAS Manager Functional Specification describes the RAS API interface.

NT RAS Manager Component Design describes internal details.

NT RAS Architecture is the overview of the entire NT RAS product.

All the above documents can be found on \\testy\consys under \src\ras\doc.

## 2.  The Problem

### 2.1.  Purpose

Device DLLs are used by RAS Manager to abstract actions required for connections according to the device type.  This modular design allows RAS Manager to support new devices by adding new DLLs.  The knowledge of the device required to make connections is contained in the DLL for that device.

The modem/X.25/switch (RASMXS) DLL will combine support for modems, X.25 pads, and switches.  These might logically be modularized in three DLLs, but are similar enough to be combined into one.  This gains us the efficiency of sharing code.

### 2.2.  Design Goals

·	Use common INF file format for all device INF files, for example, modem.inf, pad.inf, switch.inf.  This allows for a uniform set of rules for creating INF files, and a single set of APIs for reading INF files.  All device DLLs will use the Device INF File library APIs to query the respective INF files.  This API set is a wrapper above the RASFILE library.

·	Use the windows event driven model to manage the RASMXS DLL's asynchronous actions (receives).

·	Write the RASMXS DLL without specific knowledge of INF file macros wherever possible.  This means that the DLL will not have to be rewritten simply to add support for new modem AT commands.  For example, the UI and other components may need to be changed to be aware of new modem features, but there should be no need to rewrite the DLL simply to turn on and off such features.

### 2.3.  Design Exclusions

(See Section 3.3 Up-level Support.)

## 3.  Platform Support

### 3.1.  Systems

This DLL will be written to Win32 for use on Windows NT.  However, since it is intended that Win16 and  DOS versions will follow soon, this design will be made as portable as possible to those platforms.

### 3.2.  Down-level Support

The RASMXS DLL will not read RAS 1.x modems.inf files, nor provide a way to automatically upgrade the old modems.inf files for use with NT RAS.

### 3.3.  Up-level Support

·	Macros defined in INF files must be one of  the following forms.

1.	Unary macros are of the form: <macro_name>=value_string.

2.	Binary macros are of the same form with the additional requirement that macro names are composed of character strings that end in _ON or _OFF.

<NAME1>=stringA

<NAME2_ON>=stringB

<NAME2_OFF>=stringC

(For more details and examples see Appendix A.)

·	If an INF file has more than one macro definition with the same keyword, the value of the last one will be used, and no errors will be reported.

·	If a binary macro has only one of the macros defined, the other will be assumed to have the null string as is value.  For example if SPEAKER_ON>=M1 appears in the INF file but SPEAKER_OFF does not, the RASMXS.DLL will behave as though the file also contained SPEAKER_OFF>=.

·	Macros within macros are not allowed.  Macro value strings are taken literally.

## 4.  Development Tools

### 4.1.  Build Environment

The program will be built in the standard NT build environment using the BUILD command.  MS SLM will be use for source code management.

### 4.2.  Language Tools

The DLL will be written in C using the MS C8 compiler.

### 4.3  RAS Device INF File Library

A RAS Device INF File library will be developed with which to query all device INF files.  This wrapper for the RASFILE library will facilitate the translation of macros in the INF files.  The RASFILE library is being developed (adapted from the Sparta base code) within the RAS group and will be used by RASPHONE.

## 5.  The Solution

### 5.1.  Overview

The RASMXS Device DLL will implement the Device APIs for modems, X.25 pads, and switches.  The DLL will operate in the caller's context, that is it will not create threads of its own.  Most of the APIs are synchronous.  A state machine will be used to track of the steps of  the asynchronous APIs.

The asynchronous APIs will register a RAS Manager event with the system and make an asynchronous system call.  When the system completes the asynchronous action it will signal the event and RAS Manager will call DeviceWork() to advance the state machine toward completing the asynchronous action (perhaps by making a new asynchronous system call).

A linked list of data structures containing device connection information will be kept in the DLL's global memory so that it will be available over multiple calls to the DLL.

### 5.2.  Major Data Structures

#### 5.2.1  Device INF files

All INF files used with this DLL (modem.inf, pad.inf and switch.inf) must use the INF file format given in Appendix A.  Generally, INF files are of the following form.

[DeviceName]

Variables

Macros

FirstCommand

Responses

NextCommand

Responses

However, modem.inf will have a [Modem Responses] section added which will globally contain all modem responses.  Modem.inf will not normally have responses following each command.  Responses that do follow a command will be accepted before responses in the [Modem Responses] section.  This will allow for a solution in the unlikely case that some modems have conflicts with the responses in the global section.  Modem.inf files will normally be of the following form.

[Modem Responses]

Responses

[DeviceName]

Variables

Macros

FirstCommand

NextCommand

#### 5.2.2  Attribute Bits

Four attribute bits in DEVICEINFO are defined.  The existence of  a binary macro in the INF file implies that the device has a feature which is user settable.  Binary macros will have the ATTRIB_USERSETTABLE bit set.

|  | Bit | Value | Meaning |
| --- | --- | --- | --- |
|  | ATTRIB_VARIABLE | Set | Parameter is a Variable. |
|  | ATTRIB_VARIABLE | Clear | Parameter is a Macro. |
|  | ATTRIB_BINARYMACRO | Set | Parameter is a Binary Macro |
|  | ATTRIB_BINARYMACRO | Clear | Parameter is a Unary Macro |
|  | ATTRIB_USERSETTABLE | Set | Feature denoted by keyword for this attribute can be enabled or disabled by calling RasDeviceSetInfo(). |
|  | ATTRIB_USERSETTABLE | Clear | Feature cannot be enabled or disabled by user. |
|  | ATTRIB_ENABLED | Set | Feature is currently enabled. |
|  | ATTRIB_ENABLED | Clear | Feature is currently disabled. |

When the ATTRIB_VARIABLE bit is set the other attribute bits are undefined.  RasDeviceSetInfo() has no effect on any attribute bit except the ATTRIB_ENABLED bit.  Both bits are read by RasDeviceGetInfo(), and the ATTRIB_ENABLED bit may be set or cleared by RasDeviceSetInfo().  If the binary macro name appears in the DEFAULTOFF variable in the INF file, the ATTRIB_ENABLED bit will be initially clear, otherwise it will be set initially.

#### 5.2.3  Data Structures

struct DEVICE_CB {

DEVICE_CB *	pNextDeviceCB;

HANDLE	hPort;	//Port that device is on

char	szDeviceName[MAX_DEVICE_NAME];

char	szDeviceType[MAX_DEVICETYPE_NAME];

DEVICETYPE	eDeviceType;

DEVICEINFO *	pInfoTable;	//Parameters, attributes, values

DEVICEINFO *	pMacros;	//Macro names & their expansions

HRASFILE	hInfFile;	//INF file handle

HRASFILE	hModemResponses;	//Modem Responses section handle

HANDLE	hInfoTableMem;	//Memory handle for InfoTable

HANDLE	hMacrosMem;	//Memory handle for Macro Table

OVERLAPPED	Overlapped;	//Used by Win32 async file I/0

HANDLE	hNotifier;	//Event signaled when I/O done

DWORD	dCmdLen;	//Saved for a later state

char	szResponse[RAS_MAXLINEBUFLEN];

BOOL	fPartialResponse; //For slow responding modems

NEXTACTION	eDevNextAction;	//DeviceStateMachine() State

CMDTYPE	eCmdType;	//Used by DeviceStateMachine()

CMDTYPE	eNextCmdType;	//Used by DeviceStateMachine()

BOOL	fEndOfSection;	//Used by DeviceStateMachine()

RCVSTATE	eRcvState;	//ReceiveStateMachine() State

}

There are three main data structures used by the RASMXS DLL.

·	Device Control Blocks keep track of items related to each device.

·	Information Tables contains general information used when connecting a device.

·	Macro Translation Tables contains macro names and their expansions for use by RAS Device INF File APIs.

##### Device Control Block

There is one DEVICE_CB (DCB) struct for each active device.  These are kept in a linked list in the DLL's global memory so that they are available during asynchronous actions (calls).  It contains, among other items, pointers to an Information Table and a Macro Translation Table for the device.

##### Information Table

There is one Information Table for each active device.  It contains various pieces of information used to manage devices and connections.

·	Variables and raw macros are copied here from the INF file.

·	Attributes are maintained here.  RASMXS DLL infers the attribute bits from the INF file and adds them to the Information Table, and updates the attributes when called by DeviceSetInfo().

·	Phone numbers, and other information, are copied here from the UI via DeviceSetInfo().

·	Baud rates, and other information, are copied here from response strings received from the device.  This information comes via the Macro Translation Table.

The Information Table is used 1) to provide information to RAS Manager via DeviceGetInfo() calls, and 2) as source information to determine macro expansions for the Macro Translation Table.

##### Macro Translation Table

A table of macro names and their expansion strings will be used by RasDeviceGetCommand() to translate macros found in INF file command strings.  The same table will also be used by RasDevCheckResponse().  This table will be created by DeviceConnect() and DeviceListen(), and used by DeviceConnect(), DeviceListen() and DeviceWork().  There will be one table per device.

It was decided to allocate this table dynamically for two reasons.  First, the needed size is not known in until run time.  Second,  allocating it on the heap means that it can be kept across asynchronous calls but can still be freed once it is no longer needed.

### 5.3.  Pseudo-code and/or Block Diagrams

#### 5.3.1  DeviceConnect() and DeviceWork()

While this discussion focuses on DeviceConnect() and DeviceWork()the architecture of DeviceListen() will be very similar.

Receiving from the modem is handled asynchronously because there are several delays during the echo and response.  After a command is set to the modem it is echoed back with in a second or two.  Following the echo there is a longer delay until the first character of the response arrives.  Then the remaining characters arrive in short order.  This is diagrammed below.

Wait for Echo (time out ~2 Sec)

|-------------|

|

|Wait for First Character (time out ~120 Sec)

|-------------------------------------------------|

|

|Receive String

|(timeout.constant ~ 3 Sec)

|(timeout.interval ~ 500 mS)

|-----+-------------------|

|500mS|

The following pseudo code functions implement the three cases discussed above.  The asynchronous API, ReadFile(), completes when its buffer is full or it times out.  The time out could be caused by too long of an interval between characters, or too long total time to receive the string of characters.

WORD WaitForEcho() {

return(ReadFile(Timeout = 2S, BufferSize = strlen(COMMAND)));

}

WORD WaitForFirstCharacter() {

if (fListen)

Time = 0;					// 0 means infinite time out

if (fConnect)

Time = 120S;

return(ReadFile(Timeout = Time, BufferSize = 1));

}

WORD ReceiveString() {

return(ReadFile(Timeout.interval = 500mS,

Timeout.constant = 3S,

BufferSize = LARGE));

{

The DeviceStateMachine() routine is the heart of three Device DLL APIs.  DeviceConnect() and DeviceListen() will each call DeviceStateMachine() once.  DeviceWork() will call it several times.  This state machine executes command-response pairs.  In the modem case it executes the init command-response, and then the dial or listen command-response.   Receives are handled asynchronously by the ReceiveStateMachine().  DeviceStateMachine() is shown in pseudo code below.

NextAction is initialized to SEND and fEndOfSection to FALSE.  Both are reset to their initial values by DeviceDone().

RC DeviceConnect()

{

BuildMacroXlationTable();

switch(DeviceType)

{

if (NextAction != SEND || State != START)

return(ERROR_CONNECT_IN_PROCESS);

case MODEM:

CmdType = INIT;

NextCmdType = CONNECT;		//LISTEN for DeviceListen()

break;

case PAD:

case SWITCH: CmdType = GENERIC;

}

DeviceStateMachine();

}

RC DeviceStateMachine()

{

while(1)

{

switch(NextAction)

{

case SEND:

rc = RasDeviceGetCommand(&CmdType, &CommandString);

if (rc == ERROR_ENDOFSECTION)

fEndOfSection = TRUE;

NextAction = DONE;

elseif (rc == SUCCESS)

WriteFile(CommandString);

if (DeviceType == MODEM ||

RasDevResponseExpected(FilePtr))

NextAction = RECEIVE;

break;

else

Wait(2S);						//Asynchronous wait

NextAction = SEND;

break;

else

return(rc);

case RECEIVE:

rc = ReceiveStateMachine();

if (rc == SUCCESS)

NextAction = DONE;

break;

else

return(rc);

case DONE:

if (!fEndOfSection)

NextAction = SEND;		// Do next Command-Response

else

switch(CmdType)

{

case INIT:

CmdType = NextCmdType;

RasDevResetCommand();

case GENERIC:

case CONNECT:

case LISTEN:

return(SUCCESS);

}

} //End Switch

} // End While

} // End DeviceStateMachine

Each asynchronous ReadFile() call causes the state machines, and ultimetly DeviceWork(), to return PENDING.  ReadFile() is called specifing an event that is signalled when the read completes.  When the event is signaled RAS Manager calls DeviceWork() again.  This process is repeated until DeviceWork() returns SUCCESS or an error message.

extern WORD state;			//Reset to START by DeviceDone()

RC  ReceiveStateMachine()

{

do

{

switch(state)

{

case ECHO:

rc = WaitForEcho();					// Asynchronous call

if (rc)

return(rc)

else

state = FIRSTCHAR;

return(PENDING);

case FIRSTCHAR;

rc = WaitForFirstCharacter();		// Asynchronous call

if (rc)

return(rc)

else

state = RECEIVESTR;

return(PENDING);

case RECEIVESTR;

rc = ReceiveString();				// Asynchronous call

if (rc)

return(rc)

else

state = CHECKRESPONSE;

return(PENDING);

case CHECKRESPONSE:

rc = CheckResponse(&ResponseKey)

if (ResponseString == "LOOP")

state = ECHO;

break;

else

return(rc);

}

}while(state == FIRSTCHAR);

}

#### 5.3.2  DeviceEnum

WORD  DeviceEnum(

IN	CHAR *	DeviceType,

OUT	WORD *	NumEntries,

IN	BYTE *	Buffer,

INOUT	WORD *	Size)

{

GetInfFileName(DeviceType, szFileName);

RasDevEnumDevices(szFileName, NumEntries, Buffer, Size);

}

#### 5.3.3  DeviceGetInfo

DeviceSetInfo() will be very similar to DeviceGetInfo in architecture.

extern DEVICE_CB * pDeviceList;

WORD  DeviceGetInfo(

IN	PCB *	Pcb,

IN	CHAR *	DeviceType,

IN	CHAR *	DeviceName,

OUT	BYTE *	Info,

INOUT	WORD *	Size)

{

hPort = *Pcb->PCB_Handle;

FindDeviceInList(pDeviceList, hPort, &pDevice);

if (!pDevice)

{

AddDeviceToList(pDeviceList, hPort,

DeviceType, DeviceName, &pDevice);

InfoTableSize = sizeof(DEVICEINFO) + generous amount;

pInfoTable = alloc(InfoTableSize);

pDevice->pInfParams = pInfParams;

GetInfFileName(DeviceType, szFileName);

RasDevOpen(szFileName, pFilePtr);

rc = RasDevGetParams(pFilePtr, pInfTable, InfoTableSize);

if (rc) alloc larger buffer and repeat call;

//RasDevGetInfo leaves FilePtr at first command

pDevice->FilePtr = *pFilePtr;

CreateAttributes(pInfParams, pInfoTable);

}

OutputSize = CalculateOutputSize(pInfoTable);

if (Size >= OutputSize)

GetInfo(Info, pInfoTable);		//First check pInfoTable!=NULL

return(SUCCESS);

else

Size = OutputSize;

return(ERROR_BUFFER_TOO_SMALL);

}

### 5.4.  Modular Decomposition

|  | File | Contains |
| --- | --- | --- |
|  | rasmxs.c | MXS device DLL APIs |
|  | mxsutil.c | worker and utility functions |
|  | mxsstate.c | state machines, etc. |

## 6.  Testing

### 6.1.  Test Tools and Hooks

NA

### 6.2.  Unit Testing

Build a dummy DIF library if it is not completed.  Build a main that links to the RASMXS.DLL as RAS Manager would, and which calls each API once as a sanity check.

### 6.3.  Test Problems

None anticipated at this time.

## 7.  Dependencies

·	The Device INF File library and the underlying RASFILE library must be functional for the modem/X.25/switch DLL to function.

## 8. Intallation

RASMXS.DLL must be on the path.  Other DLLs that RASMXS.DLL depends on and which must also be on the path are

·	RASFILE.DLL

·	SERIAL.DLL  (the COM port media DLL)

The INF files used by RASMXS.DLL (modem.inf, pad.inf and switch.inf) must be in the %NTROOT%\RAS directory.  The entire modem.inf file may be copied to this directory, or a new modem.inf file may be created in this directory which is a subset of the modem.inf file shipped on the install floppy diskettes.  If a new modem.inf file is created it must contain at least:

·	the [Modem Responses] section

·	the modem section for each installed modem type

·	any section aliased by any of the above sections.

## 9.  Work Estimates

| Days | Task |
| --- | --- |
| 1.0 | Setup makefiles and build environment |
| 4.0 | DLL Initialization |
| 4.0 | Code data structures |
| 5.0 | Code state machines |
| 4.0 | Code Info APIs |
| 2.0 | Code Connect/Listen APIs |
| 1.0 | Code Work API |
| 1.0 | Code Done API |
| 2.0 | Code change baud rate |
| 3.0 | Unit test |
| ----- |  |
| 27 days |  |

| 1.0 | Code review |
| --- | --- |
| 1.0 | Code review changes |

## 10.  Open Issues

See ISSUE: located throughout this document.

ISSUE: What macros are needed to recognize V.42bis messages such as CONNECT/REL?  Com port baud rate will need to be changed in these cases.

ISSUE: What is the valid range of hFile?  Can NULL be used to indicate an invalid hFile?

ISSUE: Can we count on the modem to time out if there is no answer?  Must the DLL code time out after so many rings?

# Appendix A  INF Files

## 1.  INF File Format

### Definition

INF files are composed of  sections with each section devoted to a separate device.  Each section contains the following  four items.  The first three must be in the order given.   Comment Lines may appear anywhere.

|  |  | Item | Quantity |
| --- | --- | --- | --- |
|  | 1 | Section Header | Exactly One |
|  | 2 | Variable and Macro Definitions | Zero or more |
|  | 3 | Commands | One or more |
|  | 4 | Comment Lines | Zero or more |

Modem.inf has one additional section, a global [Modem Responses] section that contains valid responses for all modems.  Responses for pads and switches, on the other hand, immediately follow their associated commands.  There is no global response section in pad.inf or switch.inf.

Section Headers

Section headers identify the specific device to which the section applies.  In modem.inf it is normally a name which identifies the modem make and model.  A section header is a string of  0 to N characters between square braces.  (N is defined by RASFILE library.)  For example:

[WizBang UltraFine]

Variable Definitions

Variables are used to pass information from the INF file to the device DLL.  Variables are not substituted into command or response strings as macros are.  Variable definitions must come before the first command, and by convention come before the macro definitions.  Variables definitions are of the form variable_name=value_string.  For example:

MAX_CARRIER_BAUD=9600

CALLBACK_TIME=8

DEFAULTOFF=

Macro Definitions

Macros are place holders in command strings.  These place holders are replaced at command time with the actual command characters.  Macro definitions must come before the first command, and by convention come after the variable definitions.  There are two types of macros, unary and binary.  Macro names must be enclosed in angle brackets (<>).

1.	Unary macros are of the form: <macro_name>=value_string.

2.	Binary macros are of the same form with the additional requirement that macro names end with _ON> or _OFF>.

For example:

<RESET>=&F

<SPEAKER_ON>=M1

<SPEAKER_OFF>=M0

The command AT<reset><cr> would be sent as AT&F0D.  Binary macro place holders are replaced according to instructions from the user.  For example, if the user disables the speaker then the command AT<speaker><cr> would be sent as ATM00D.  If the user enables the speaker then it would be sent as ATM10D.

Some responses may also use macros.  Most macros in response strings behave the same way as in commands.  However, certain macros are used to capture information such as baud rate from the device response string.  Examples are <baudrate> and <x25address>.

Nested macros are not allowed.  A left angle bracket without a matching right angle bracket within N characters will be interpreted as a less-than sign (<) and not as a macro symbol.  Similarly, a right angle bracket without a matching left angle bracket preceding it will be interpreted as a greater-than sign (>).  Two adjacent left brackets will always be interpreted as a greater-than sign, and two adjacent right brackets will always be interpreted as a less-than sign.  This allows for the case where these symbols are required in a command string.

Commands

Commands are strings of characters sent to the device.  These strings may contain macros.  Commands are of the form: command_keyword=value_string.  The set of command_keywords is {COMMAND, COMMAND_INIT, COMMAND_DIAL, COMMAND_LISTEN}.  Modem.inf uses command types COMMAND_INIT, COMMAND_DIAL, and COMMAND_LISTEN to initialize the modem, dial, and put the modem in answer mode respectively.  Pad.inf and switch.inf use the command type COMMAND.  Commands of a given type are executed in the order found in the INF file.  By convention commands of  the same type are grouped together.  Modem.inf example:

COMMAND_INIT=AT&F S0=0 <speaker><v42><v42bis><flowcontrol><cr>

COMMAND_LISTEN=ATS0=1<cr>

COMMAND_DIAL=ATDT<phonenumber><cr>

Responses

A Command-Response Set consists of one command followed by zero or more responses.  Responses are strings that are expected to be received from the device, and may contain macros.  Responses are of the form: keyword=value_string.  Modem.inf responses are placed in the global [Modem Responses] section.  But pad and switch responses are closely associated with commands.  Responses for a given command must follow that command and come before the next command.

Modem.inf example:

[Modem Responses]

LOOP=<cr><lfRING><cr><lf>

CONNECT=<cr><lfOK><cr><lf>

ERROR=<cr><lfERROR><cr><lf>

Pad.inf example:

COMMAND=C<x25address><cr><lf>

CONNECT=<match>"CONNECT"

ERROR=<match>"ERROR"

LOOP is a special response.  Normally when the DLL receives a string that matches one of the expected responses, it moves on to the next command.  However, if the matched response is LOOP, it receives the next string from the device and again tries to find a match from the same set of responses.  As long as the matched response is LOOP it will not move on the next command-response set.  Any response key_word that begins with LOOP_ has this same behavior.

Comment Lines

Comment Lines begin with a semicolon (;) and may appear anywhere in the file.  Comment Lines are ignored by RASFILE library APIs.  Comment Lines are useful as notes to people who maintain the INF files.  For example:

; Explanation of modem commands

; &F  Reset modem to factory default settings

; &C1 DCD tracks presence of modem carrier

; &D2 Hangup & disable autoanswer when DTR goes from ON TO OFF

### Example modem.inf

The following is an example of what a hypothetical modem.inf file would look like.

;------------------------------------------------------------------

[Modem Responses]

;

; All modem responses normally appear here.  All modems check for

; responses in this global list.

;

LOOP=<cr><lf>RINGING<cr><lf>

CONNECT=<cr><lf>PLEASE SET TO <carrierbaud>

CONNECT=<cr><lf>CONNECT <carrierbaud><cr><lf>

CONNECT=\

<cr><lf>CARRIER <carrierbaud><cr><lf><append>\

<cr><lf>PROTOCOL: NONE<cr><lf>\

<cr><lf>CONNECT <connectbaud><cr><lf>

CONNECT_V42=\

<cr><lf>CARRIER <carrierbaud><cr><lf><append>\

<cr><lf>PROTOCOL: V.42/LAPM<cr><lf>\

<cr><lf>COMPRESSION: NONE<cr><lf>\

<cr><lf>CONNECT <connectbaud><cr><lf>

CONNECT_V42BIS=\

<cr><lf>CARRIER <carrierbaud><cr><lf><append>\

<cr><lf>PROTOCOL: V.42/LAPM<cr><lf>\

<cr><lf>COMPRESSION: V42BIS<cr><lf><cr>\

<lf>CONNECT <connectbaud><cr><lf>

ERROR=<cr><lf>ERROR<cr><lf>

ERROR_BUSY=<cr><lf>BUSY<cr><lf>

ERROR_NO_ANSWER=<cr><lf>NO ANSWER<cr><lf>

ERROR_NO_CARRIER=<cr><lf>NO CARRIER<cr><lf>

ERROR_NO_DIALTONE=<cr><lf>NO DIALTONE<cr><lf>

;------------------------------------------------------------------

[WizModem UltraFine]

;

; Macros and Variables must come before the first command.

; By convention Macros come before Variables.

;

; These are reserved macro names which are defined implicitly and

; may not be redefined here: <cr> <lf> <blank> <phonenumber>

; <x25address> <x25diagnostics> <match> <ignore> <baudrate>

;

; Typically a modem hardware error correction protocol must be

; enabled in order to use modem hardware compression.

;

<carrierbaud>=

<connectbaud>=

<message>=

<phonenumber>=

<hdwflowctrl_off>=&K0

<hdwflowctrl_on>=&K4

<protocol_off>=&Q0 S36=1

<protocol_on>=&Q5 S36=5 S46=136

<compression_off>=

<compression_on>=&Q5 S36=5 S46=138

<speaker_off>=M0

<speaker_on>=M1

CALLBACK_TIME=10

DEFAULTOFF=protocol compression hdwflowctrl

MAX_CARRIER_BAUD=9600

MAX_CONNECT_BAUD=19200

;

; Commands must be in order.  By convention commands of the

; same type are grouped together.  Command types for modems are

; COMMAND_INIT, COMMAND_DIAL, and COMMAND_LISTEN.  (All other devices

; use command type COMMAND.)

;

; Modem responses are normally located in the global

; [Modem Responses] section.  In extaordinary cases a modem response

; may appear here immediately following its command.  Such responses

; are checked before the global responses.

;

COMMAND_INIT=AT&F &C1 S0=0<speaker><protocol><compression> <flowcontrol><cr>

COMMAND_LISTEN=ATS0=1<cr>

COMMAND_DIAL=ATDT<phonenumber><cr>

;-------------------------------------------------------------------

; Modems which are 100% compatible with another modem which has

; a full entry in this file, may refer to that other modem as an

; ALIAS.  The aliased entry will then be used.  Aliases to aliases

; are not allowed.

;

[QT Modem]

ALIAS=WizModem UltraFine

## 2.  Macros

Macros which are defined in code are considered reserved words and may not be redefined in INF files.  Macros which are defined in INF files and rasinf.h maybe redefined by editing the pertinent INF file and rasinf.h.

|  | Defined in Device INF API code | May be used in |
| --- | --- | --- |
|  | cr | Commands and Responses |
|  | lf | Commands and Responses |
|  | match | Responses Only |
|  | w | Responses Only |
|  | append | Responses Only |
|  | hXX (XX are hexadecimal digits) | Responses |

|  | Defined in INF file & RASMXS.H | May be used in | Used for |
| --- | --- | --- | --- |
|  | phonenumber | Commands Only | Modems |
|  | speaker | Commands Only | Modems |
|  | protocol | Commands Only | Modems |
|  | compression | Commands Only | Modems |
|  | hdwflowcontrol | Commands Only | Modems |
|  | x25address | Commands Only | X.25 |
|  | userdata | Commands Only | X.25 |
|  | carrierbaud | Responses Only | Modems |
|  | connectbaud | Responses Only | Modems |
|  | x25diagnostics | Responses Only | X.25 |
|  | message | Responses Only | Any |

### Macros defined in Device INF API code

The w macro is a wild card which matches any one character, for example, CO<w><w>2 matches COOL2 or COAL2 but not COOL3.  The match macro includes a quoted argument string.  If the string is found anywhere in the device response a match is reported.  For example, if the following response is defined possible matches are shown:

RESPONSE1=<match>"AxByCz"

RESPONSE2=shrimp boat<match>"sails"

|  | 187AxByCzD876543 | Matches RESPONSE1 |
| --- | --- | --- |
|  | A line of text ending in AxByCz | Matches RESPONSE1 |
|  | AxBy | Does not match |
|  | shrimp boat with red sails | Matches RESPONSE2 |
|  | shrimp boat reefing blue sails | Matches RESPONSE2 |
|  | boat sails | Does not match |

The cr and lf macros are convenient ways to enter carriage return and line feed characters into INF files.  The hXX macro allows any hexadecimal character to appear in a string including the zero byte, <h00>.  The append macro is discussed under RasDevCheckResponse in Appendix B.

### Macros defined in INF file and RASMXS.H

Response macros are used in responses to capture information returned from the device.  For example, the macro carrierbaud is defined in MacroXlations by modems.dll to be the null string, "".  This value field is filled in by RasDevCheckResponse with the baud rate substring received from the modem.

The message macro is used to return response keys to the caller (UI).  The INF file should not have any response values that include the message macro.  This macro is also used to return unrecognized response strings from the device.  When the device responds with a string that cannot be matched to an expected response string in the INF file, the error code ERROR_UNRECOGNIZED_RESPONSE is returned to the caller,  and the device response is put into the InfoTable as the value for the message key.  A call to DeviceGetInfo will retrieve the most recent response key, or unexpected raw response string.

ISSUE: What macros are needed to recognize V.42bis messages such as CONNECT/REL?  Com port baud rate will need to be changed in these cases.

### Return Codes and Macros

There are three return codes from DeviceWork which indicate device responses.

1.	SUCCESS

2.	ERROR_FROM_DEVICE

3.	ERROR_UNRECOGNIZED_RESPONSE

The first two indicate that the response returned from the device was found in the device INF file.  When SUCCESS is return the keyword for the matched response begins with CONNECT or OK.  ERROR_FROM_DEVICE indicates that the keyword begins with ERROR.  These keywords are placed in the value field for the 'message' keyword, and can be retrieved by calling DeviceGetInfo.

The third indicates that the response was not found in the device INF file, and the raw response is placed in the value field of the 'message' keyword.  This allows the UI to display something more meaningful that ERROR_UNRECOGNIZED_RESPONSE when unexpected things happen.

# Appendix B   RAS Device INF File APIs

The RAS Device INF File Library is wrapper above the RASFILE library.  A main purpose of this library it to facilitate the expansion of macros found in the INF files.  Appendix A contains details on INF files and macros.

All knowledge of  macro delimiters (presently '< 'and '>') is encapsulated in the Device INF File Library.  For example, RasDevGetParams will return macro names without the delimiters, and the DEVICEINFO table which is an input to RasDevCheckResponse will contain macro names without delimiters.

DWORD APIENTRY  RasDevEnumDevices(

IN	TCHAR *	lpszFileName,

OUT	WORD *	pwNumEntries,

OUT	BYTE *	pBuffer,

IN OUT	WORD *	pwSize)

This function returns in *pBuffer an array of DEVICE structs which contains all the devices in the INF file, that is, it returns all of the header names in the file.  The attributes field is undefined.  If the buffer supplied is not big enough the function returns ERROR_BUFFER_TOO_SMALL and the size of the required buffer is put in the pwSize parameter.  *pwNumEntries is the number of DEVICE structs returned.  This function is a simple interface to a RASFILE API.

Return values:

SUCCESS

ERROR_FILE_COULD_NOT_BE_OPENED

ERROR_BUFFER_TOO_SMALL

DWORD APIENTRY  RasDevOpen(

IN	TCHAR *	lpszFileName,

IN	TCHAR *	lpszDeviceName,

OUT	HRASFILE *	phFile)

This function is used to open an INF file section for use by the device DLL.  For example, for a modem RASMXS.DLL will call RasDevOpen to open a section of modem.inf.  On return hFile is a file handle to a memory image of the section whose name is szDeviceName.  The file is opened in read only mode.  The underlying RASFILE function will result in a new memory image of the specified section each time it is called.   If the same or another section has already been opened open, RasDevOpen returns a file handle to a new memory image leaving existing one(s) unchanged.

Return values:

SUCCESS

ERROR_FILE_COULD_NOT_BE_OPENED

ERROR_DEVICENAME_NOT_FOUND

ERROR_DEVICENAME_TOO_LONG

void APIENTRY  RasDevClose(

IN	HRASFILE	hFile)

This function is used to close an INF file section used by the device DLL.

ISSUE: What is the valid range of hFile?  Can NULL be used to indicate an invalid hFile?

ISSUE:  Should return type of RasDevClose be void?

DWORD APIENTRY  RasDevGetParams(

IN	HRASFILE	phFile,

OUT	BYTE *	pBuffer,

IN OUT	DWORD *	pdSize)

This function returns in *pBuffer a DEVICEINFO struct which contains all the keyword=value pairs between the top of the section to the first command.  Keywords which are macros have the macro delimiters ('<','>') removed.  The ATTRIB_VARIABLE bit will be set if the keyword is a variable, otherwise it is cleared.  All other attribute bits are always cleared.  If the buffer supplied is not big enough the function returns ERROR_BUFFER_TOO_SMALL and the size of the required buffer is put in the pdSize parameter.  On return CurLine points to the first command in the section.

On return the following is true of the data in pBuffer.  (Refer to Section 3.3.)

1.	Macros and variables are sorted in aphabetic order by keyword.

2.	Every macro_ON has a matching macro_OFF.

3.	There are no duplicate keywords.

Return values:

SUCCESS

ERROR_BUFFER_TOO_SMALL

DWORD APIENTRY  RasDevGetCommand(

IN	HRASFILE	hFile,

IN	CMDTYPE	eCmdType,

IN	DEVICEINFO *	pMacroXlations,

OUT	TCHAR *	lpszCommand)

If CurLine (see RASFILE functional specification) currently points to a command and the command type matches eCmdType,  the command on this line is returned.  Otherwise the file pointer is advanced to the next command matching the given command type, and that command is returned.  The type of the command may be GENERIC, INIT, DIAL, or LISTEN.  Any macros found in the command string are expanded using the translation table in *pMacroXlations.  If a macro is found in the command string in the INF file but not found in *pMacroXlations then ERROR_MACRO_NOT_DEFINED is returned.  On return CurLine points to the next line following the command returned.  The next line will be the first response, or the following command (or the end of the section) if there is no response .  CMDTYPE  is an enum and is defined in rasmxs.h.

Return values:

SUCCESS

ERROR_END_OF_SECTION				// Implies no more commands

ERROR_MACRO_NOT_DEFINED

DWORD APIENTRY  RasDevResetCommand(

IN	HRASFILE	hFile)

This function moves CurLine to the first command of any type in the section.  It is used after doing all commands of a given type (for example, INIT) before doing the next type of command (for example, DIAL).

Return values:

SUCCESS

ERROR_NO_COMMAND_FOUND

DWORD APIENTRY  RasDevCheckResponse(

IN	HRASFILE	hFile,

IN	TCHAR *	lpszReceived,

IN	DWORD	dReceivedLen,

IN	DEVICEINFO *	pMacroXlations,

OUT	TCHAR *	lpszResponseKey)

This function returns in *lpszResponseKey the keyword found in the line whose value matches the string received from the modem (passed in *lpszReceived).  Fixed macros (such as <cr> and <lf>) found in the value string are expanded before a match is tried.  If the a baud rate macro appears in the value string, the actual baud rate substring will be copied from the received string into pMacroXlations as a value for the appropriate keyword.  The x25diagnostics macro is handled in the same manner.  dReceivedLen is the length in bytes of the device raw response which is in lpszReceived.  The received data may contain zeros; a zero byte should not be assumed to be a string terminator.

When the append macro appears in a response RasDevCheckResponse attempts to

1.	Match the entire string with <append>="", and return SUCCESS.

2.	Match up to <append>, and return ERROR_PARITAL_RESPONSE.

3.	Return ERROR_UNRECOGNIZED_RESPONSE .

RasDevCheckResponse will check all responses in a Command-Response Set in order and return the first match found.  If when the function is called CurLine points to a command or the end of the section, the function returns ERROR_NO_RESPONSES.  If after checking all responses no match is found, the function returns ERROR_UNRECOGNIZED_RESPONSE.

Return values:

SUCCESS

ERROR_PARTIAL_RESPONSE

ERROR_MACRO_NOT_DEFINED

ERROR_UNRECOGNIZED_RESPONSE

ERROR_NO_RESPONSES

BOOL APIENTRY  RasDevResponseExpected(

IN	HRASFILE	hFile)

This function checks the INF file for the presence of responses to the current command.  RasDeviceGetCommand  must be called before this function is called.  If the CurLine points to a Command the function returns FALSE, otherwise it returns TRUE.  CurLine is unchanged by this call.  Since the [Modem Responses] section will never be empty, if hFile points to a modem.inf file this function always returns TRUE.