Windows NT Remote Access

RASBATCH

Component Design

Revision History

| Revision | Date | Author | Comment |
| --- | --- | --- | --- |
| 0.1 | 05/12/92 | Steve Cobb | Original draft |

Table Of Contents

µ1.  Audience	3

1.1.  Prerequisites	3

1.2.  References	3

2.  The Problem	3

2.1.  Purpose	3

2.2.  Design Goals	3

2.3.  Design Exclusions	3

3.  Platform Support	3

3.1.  Systems	3

3.2.  Down-level Support	4

3.3.  Up-level Support	4

4.  Development Tools	4

4.1.  Build Environment	4

4.2.  Language Tools	4

4.3.  RASFILE	4

5.  The Solution	4

5.1.  Overview	4

5.2.  Major Data Structures	4

5.3.  Pseudo-code and/or Block Diagrams	4

5.4.  Modular Decomposition	4

6.	Testing	5

6.1.  Test Tools and Hooks	5

6.2.  Unit Testing	5

6.3.  Test Problems	5

7.  Dependencies	5

8.  Work Estimates	5

9.  Open Issues	5

# 1.  Audience

## 1.1.  Prerequisites

The reader is assumed to be familiar with the following:

·  NT RAS product goals and terminology

·  RAS 1.0 RASDIAL for DOS/OS2 functionality.

·  RAS Manager APIs functional specifications.

·  RAS External APIs functional specifications.

## 1.2.  References

NT RAS Windows RASPHONE Functional Specification describes the functional requirements of Windows RASPHONE.  See \\testy\consys\src\ras\doc\rasphofs.doc.

Windows RASPHONE Visual Basic prototype demonstrates the operation and appearance of Windows RASPHONE on dummy data.  See \\testy\consys\src\ras\proto\rasphone\rasphone.exe.

NT RAS Architecture is an architectural overview of the entire NT RAS product.  See \\testy\consys\src\ras\doc\rasarch.doc

NT RAS External API Functional Specification and Component Design describes the external APIs which provide developers with dial/hangup/status functionality from within a program.  See \\testy\consys\src\ras\doc\funcspec\rasapifs.doc and \\testy\consys\src\ras\doc\design\rasapicd.doc.

NT RAS RAS Manager Functional Specification and Component Design describes the RasXxx API interface.  See \\testy\consys\src\ras\doc\funcspec\rasmanfs.doc and \\testy\consys\src\ras\doc\design\rasmancd.doc.

NT RAS RASFILE Functional Specification and Component Design describes the configuration file editing library.  See \\testy\consys\src\ras\doc\funcspec\rasfilfs.doc. and \\testy\consys\src\ras\doc\design\rasfilcd.doc.

# 2.  The Problem

## 2.1.  Purpose

See functional specification.

## 2.2.  Design Goals

·	Provide easy-to-use command line access to the RAS connection management functions.

## 2.3.  Design Exclusions

·	The user must create entries with RASPHONE.  RASBATCH is not meant to duplicate all functionality of RASPHONE, but only that which makes sense from a batch file.

·	Interactive Terminal mode is not supported.

# 3.  Platform Support

## 3.1.  Systems

See functional specification.

## 3.2.  Down-level Support

RASBATCH cannot read RAS 1.0 phone books until they have been upgraded (automatically) by running RASPHONE.  An appropriate error message is displayed in this case.

## 3.3.  Up-level Support

The new /DEVICE option is fairly open-ended with the intent of supporting parameters on new device types as they become available.

# 4.  Development Tools

## 4.1.  Build Environment

The program will be built in the standard NT build environment using the BUILD command rather than makefiles.  The MS SLM program will be used for source code management.

The source trees will be set up with the intent of building both DOS and Win32 versions of the program.  The anticipated source tree location of this component is \\testy\consys\ras\src\ui\client\batch.

ISSUE: Verify that NT build environment supports Win16 build using Win 3.1 SDK tools.

## 4.2.  Language Tools

COW RASPHONE will be based on MS C 7.0, Win32 RASPHON on MS C 8.0.

## 4.3.  RASFILE

The RASFILE library (based on the Sparta CFG file editor) will be used to manipulate the PHONE.INF file contents.

# 5.  The Solution

## 5.1.  Overview

RASBATCH will be use the existing DOS RASDIAL parsing code with minor changes to support /DISCONNECT without an entry and /DEVICE.  The back-end code will be changed to call the External RAS API calls.   So, basically there's not much new code in RASBATCH and I'm not going to waste time duplicating the RAS 1.0 spec.

## 5.2.  Major Data Structures

See existing RASDIAL code.

## 5.3.  Pseudo-code and/or Block Diagrams

See existing RASDIAL code.

## 5.4.  Modular Decomposition

Will logically extend existing RASDIAL code.

# 6.	Testing

## 6.1.  Test Tools and Hooks

## 6.2.  Unit Testing

## 6.3.  Test Problems

# 7.  Dependencies

·	External RAS APIs

·	RASFILE library

# 8.  Work Estimates

| Days | Task Description |
| --- | --- |
| 2.0 | Set up DOS build environment |
| 3.0 | Remove WANTSR-style connection stuff and all OS2 junk |
| 1.0 | Change parsing code |
| 1.0 | Call external RAS APIs |
| 3.0 | Unit test DOS version |
| 1.0 | Set up Win NT environment |
| 1.0 | Add Win NT specific ifdefs |
| 1.0 | Unit test Win NT version |
| 1.0 | Code review |
| 1.0 | Code review changes |
| ----- |  |
| 15.0 |  |

# 9.  Open Issues

See ISSUES: items throughout.