Asynchronous MAC Functional Specification

# LAN Manager Remote Access Services 2.0

Connectivity & Systems Integration Group

Authors: Thomas J. Dimitri

Revision History

Revision	Date			Author		Comment

0.1		March 25, 1992		tommyd	Original

0.2		May 8, 1992		tommyd	Changes Requested by Review

Table of Contents

µIntroduction	3

Description	3

Organization of Document	3

Intended Audience	3

Goals	3

Related Components and Documents	3

Document Conventions	3

Overview	4

Meat of the Functional Specification	6

AsyncMAC IOCtl APIs	6

Description	7

Handling multiple protocols from one client.	8

SLIP Framing	9

# Introduction

## Description

The Asynchronous MAC (AsyncMAC) is an NDIS 3.0 device driver layered between the RasHub and a serial driver.  The serial driver is any NT device driver which conforms to the Win32 and Nt communication driver specification.  The AsyncMAC is similar to an ethernet MAC in that it is responsible for encapsulating a frame with its own header and CRC, transmitting it, and then decapsulating it and verifying the CRC upon reception.  The AsyncMAC also has hooks for compression.

## Organization of Document

This documents is ordered in the following format: An overview, followed by a list of the APIs, a description of the APIs, then how to handle multiple protocols from one client.

## Intended Audience

The intended audience for this document is anyone who wishes to understand the NT RAS subsystem, the async frame format, or the compression hooks in the AsyncMAC.  Some knowledge of how the RasHub works or how an NDIS 3.0 MAC may be written can be derived from this document as well.

## Goals

The AsyncMAC should be able to handle an unlimited number of connections if resources are unlimited.  The architecture should support simultaneous thread execution so that performance improves on a MultiProcessor platform (i.e. the architecture should not have one thread waiting on multiple events and thus serializing events at the thread level).  The AsyncMAC should not use any special IOCtl controls to serial device drivers.  Performance should be equal to or better than that of RAS 1.0 on OS/2 (with the same hardware).  Serial driver I/O calls should be made at kernel level.  The AsyncMAC should port to "Cougar" with not much effort as all the code should be written in 'C' and conform to the NDIS 3.0 specification.  Multiplexing should be very fast.

## Related Components and Documents

The NT RAS Archiecture document should be read before reading this functional specification.  The NDIS 3.0 document describes the upper level interface of the AsyncMAC.  The RasHub, Compression, and Device DLL functional specifications are all related to the AsyncMAC.  Knowledge of NT serial IOCtls is also needed.

## Document Conventions

Standard stuff.

# Overview

The AsyncMAC is an NDIS 3.0 MAC driver wedged between the RasHub (which looks like an NDIS transport to the AsyncMAC) and one or more serial device drivers.  The AsyncMAC only has one NDIS binding with the RasHub.

µ §

In the figure above, there is one NDIS binding from the AsyncMAC to the RasHub (note that the RasHub may bind to other MACs as well).  The AsyncMAC, after connection occurs, gets a FILE handle to NtWriteFile, NtReadFile APIs.   The FILE handle is obtained from the calling process (probably the modem.dll) which has already opened the serial device driver.  Thus, the AsyncMAC is not "device layered" above the serial device drivers (i.e. it does not communicate to the serial device drivers directly via IRPs).  Instead, it goes through the I/O manager which generates IRPs to the serial device driver.  The AsyncMAC also provides an IOCtl interface for dynamic connections and disconnections.  The AsyncMAC is NOT in control of the serial port when a modem is being initialized.  At this point, the modem device DLL controlled by RasMan is in control of the serial port.

The AsyncMAC does not use IRPs for several reasons:

1. IRPs are not portable to non-NT platfroms such as 'Cougar'.

2. By using IRPs instead of going through the I/O system use of a redirected serial port is impossible.

3. The Serial IRPs are difficult to use a (according to TonyE) and probably wouldn't even work becuase the AsyncMAC would be layered on top of the serial driver and thus if the serial driver were to be relinquished to a user mode app, it might not work (TonyE says there are complications here as well).

The AsyncMAC has three interfaces.  One is a device driver IOCtl interface to allow the RasManager device DLLs to Open and Close connections with serial ports.  The second is the NDIS interface with the RasHub. The third is the NT IO interface to the serial device driver.  The AsyncMAC may also optionally have a compression library it calls out to.

Through these interfaces the AsyncMAC accomplishes taking and NDIS ethernet frame and shipping across the serial port (possibly compressing it) and doing the reverse, receiving a frame from the serial port (possibly decompressing it) and sending up to RasHub.

The AsyncMAC uses system threads to read and write to the serial port.  The AsyncMAC also keeps up to date the statistics (bytes rcvd, transmitted, data overruns, queue overruns, timeouts, CRC errors, frames rcvd, frames sent).  Most of the AsyncMACs work is performed at APC level (not DPC level like most NDIS dirvers).

µ §

# Meat of the Functional Specification

## AsyncMAC IOCtl APIs

There are only three IOCtls supported by the AsyncMAC.  These three APIs cover all that 	the AsyncMAC needs to frame and deframe packets over the serial device driver.

AsyncMAC to RasMan IOCtl APIs

AsyncMacOpen(IN hRasEndpoint, IN FileHandle,  IN LinkSpeed,

IN QualityOfConnection, OUT CompressionVersionsSupported)

AsyncMacClose(IN hRasEndpoint)

AsyncMacCompress(IN hRasEndpoint, IN CompressionVersionWanted)

hRasEndpoint is a 32 bit handle which should not exceed 64,000 (i.e. 16 bits in size).  The hRasEndpoint must unique for each endpoint.  This unique handle is the same handle that RasHub acts on.  The 6 byte ethernet address of the endpoint will be "DESTxx" where xx is the 16 bit hRasEndpoint.  This of course limits the number of possible endpoints to 64,000.

FileHandle is the 32 bit FileHandle returned on the Win32 CreateFile or OpenFile.  The Win32 handle is the same as the Nt handle.  The AsyncMAC uses the Nt file handle to write and read from the serial port.

LinkSpeed is a 32 bit unsigned integer in bits per second.  This is the RAW speed not the optimal compressed speed.  That is, if modem has V.32 and makes a V.42bis connection on top of the V.32 (9600) connection, the RAW speed is 9600.  The LinkSpeed is passed up to real NDIS protocols so they can dynamically adjust their timeouts.

QualityOfConnection is a 32 bit unsigned integer.  Currently NDIS must better define what it wants fmor this field.  I suspect something like 0-raw mode, 1-V.42(bis), 2-X.25

CompressionVersionsSupported is a 32 bit field.  A one set in the bit field indicates that that version of compression is supported for this endpoint.  For example, all 0s would indicate that the AsyncMAC does not support any type of compression.  A 1 in the least significant bit (bit 0) would indicate that version 1 compression is supported.  A 1 in bit 2 and bit 0 would indicate that version 3 and version 1 compression is supported, but not version 2.  Compression versions are thus limited to 32 versions.  New versions may exist as dictionaries or algorithms are improved.

CompressionVersionWanted is a 32 bit field with bit sets indicating which compression versions may be used on this endpoint.  It is possilble that in the future, two or more compression routines will be used (like Huffman and LZRW3).  Having the ability to use either can make compression better.  No bit can be set that is not set in the CompressionVersionsSupported field.  If no bits are set, no compression will be used.

## Description

The AsyncMAC gets called with AsyncOpen when the serial line is connected from the client to the server (past modem init and modem dialing or auto-answering stages).  The FileHandle is used to send and receive frames.  The LinkSpeed in conjunction with QualityOfService is used to determine FRAME timeouts and to update the transport on the transport retransmission timeouts and retries.  The QualityOfService is the same as the NDIS 3.0 field.  X.25 and V.42bis have a higher QualityOfService than direct modem connections.

ISSUE:  Do we need to know the intercharacter timeouts?  Do we need to set a wait-for-something timeout?

# Handling multiple protocols from one client.

With the advent of LanMan 2.1 Novell Client connectivity, a RAS Client may want to use both IPX and AsyBEUI as RAS transports to talk to both Novell and LanMan 2.1 Servers.  This means that a RAS client may be using two protocols over the AsyncMAC.  This can easily be done by NT RAS.  The key is in the frame format.  AsyBEUI uses the length field in 802.3, IPX and IP use a specific ethernet type to distinguish themselves on a heterogeneous network.  The figure below depicts the frame format for different protocols.

µ §

So RasHub can tell by looking at the 802.3 Length/Type field what protocol to pass the frame up to.  The AsyncMAC formats the frame differently for Length (saving 2 bytes) then it does for transports which use the Type field.

For compressed frames the address field may change to include a bit which tells the AsyncMAC if the frame is compressed or not.

# SLIP Framing

#define ENDCHAR		192

#define ESCCHAR		219

#define ESCENDCHAR	220

#define ESCESCCHAR	221

Each frame begins with the ENDCHAR and ends with ENDCHAR.  Any byte in the middle which an ENDCHAR or ESCCHAR is changed to a two byte representation to ESCCHAR ESCENDCHAR or ESCCHAR ESCESCCHAR respectively.

1006 bytes is the largest frame typically accepted.  This means that the IP protocol probably has to ship frames of less than 1500 bytes.  Thus the SLIP mac should defined the maxframesize to be 1006.