# Revision 7, 26 May 96

80% complete.

# Revision History

1,2,3	-		First drafts.

4	26may96	First update cycle

Changes

Too Many

5	4jun96		Second update cycle

- CPUs are to be named \CPU.x, i.e. \CPU.0  \CPU.1   \CPU.31 ...

- Instance scope is gone.

# Introduction:

This document contains formal descriptions of two languages.

The first is the ACPI Control Method virtual machine language.  This is a machine code for a virtual machine which is supported by an ACPI compatible operating system, and in which ACPI control methods are written.  Humans do not normally write this language.  This language is called AML, the ACPI control method Machine Language.

The second language is a source language for writing ACPI Control Methods.  Humans write this langauge and use a translator (a compiler) to generate AML from it.  This language is called ASL, or the ACP control method Source Language.

All ACPI compatible OSes must support AML.  However, a given user might define some aribtrary source language (to replace ASL) and write a tool to translate it to AML.  However, the ACPI group will support a single translator for a single language , ASL.

An OEM or BIOS vendor needs to write ASL and be able to single step AML for debugging.  (Debuggers and like tools are expected to be AML level tools, not source level tools.)  An ASL translator implementor must understand how to read ASL and generate AML.  An AML interpreter author (an OSV) must understand how to execute AML.

AML and ASL are different languages though they are closely related.

### Conventions:

xdd, such as x03, refers to a byte value expressed as two hexadecimal digits.  So the C constant 0x21 is written in AML (and ASL) as x21.

Words in bold are the names of terms in the grammar, representing any instance of such a term.

Words in bold(arg) followed by an argument in paranthesis represent a particular instance of some term, with a particular value.

Thus, ByteConst(3) refers to an 8 bit constant with the value 3.   And DefString(LX35) would declare a string ‘LX35’.

Words in italic are the names of arguments to objects that are replaced for a given instance.  Thus ByteConst(bytevalue)

term => representation shows the actual byte representation for the term.  Thus, we would have byteconstant(3) => x03.

Single quotes (‘’) are used to indicate constant characters, for example ‘A’ => x41.

Terms may be defined as sequences of other terms using :=.  So aterm := bterm bterm means that any aterm may be expanded to bterm bterm.

Terms that follow each other by spaces are simply ordered lists.

[] are used to bracket terms into a group, as in [a b] | [c d] meaning either a b or c d.

The bar symbol ‘|’ separates alternatives.

aterm := bterm | [cterm dterm] allows for:

bterm cterm

bterm dterm

aterm := [bterm | cterm] dterm allows for:

bterm dterm

cterm dterm

Ranges are expressed with -, so 1-9 is a single digit in the range 1 to 9 inclusive.

A superscript following a term is a repeat count.  So aterm3 is equivalent to aterm aterm aterm.

term:n refers to an n bit field.

|= is bitwise OR of terms on the right, normally bit fields (like term:n), it’s used in place of :=.

flagterm:term refers to the term bitfield within flagterm.

# AML: ACPI Control Method Machine Language

AML is the language processed by the ACPI method interpreter.  It is a largely declarative language.  It’s best not to think of it as a stream of code, but rather as a set of declarations that the ACPI interpreter will compile into the ACPI name space at definition block load time.  For example, notice that DefByte allocates an anonymous integer  variable with a byte size initial value in ACPI space, and passes in an initial value.  The byte in the AML stream that defines the initial value is NOT the address of the variable’s storage location.

ByteConst =>  x00 - xff

WordConst => x0000 - xffff

DWordConst => x00000000 - xffffffff

AsciiChar => x01 - x7f - any standard ascii character except for null

NullChar => x00

length(expression)

The length, in bytes, of expression.  Used with PkgLength.  See DefAnonPkg.

PkgLength := PkgLead ByteConstfollowcount

PkgLead |= [6-7: followcount => 1-3

4-5: reserved

0-3: 0-15] |

[6-7: followcount => 0

0-6: 0-63 ]

The high 2 bits of the first byte reveal how many follow bytes are in the PkgLength.

If PkgLength itself is more than 1 byte long (value > 63) then only 4bits are used from the first byte.  Max value is 3*8 + 4 = 28 bits.  The value is little endian.  See DefAnonPkg.

Examples:

PkgLength(63) => 0x3f

PkgLength(64) => 0x40 0x04

PkgLength(xf13ba4) => 0xc4 0xba 0x13 0x0f

Name := NameTail |

[RootPrefix NameTail] |

[ParentNameHead NameTail]

NameChar => ‘A’ | ‘B’ | ‘C’ | ‘D’ | ‘E’ | ‘F’ | ‘G’ | ‘H’ | ‘I’ | ‘J’ | ‘K’ | ‘L’ | ‘M’ | ‘N’ | ‘O’ | ‘P’ | ‘Q’ | ‘R’ | ‘S’ | ‘T’ | ‘U’ | ‘V’ | ‘W’ | ‘X’ | ‘Y’ | ‘Z’ |  ‘_’ | ‘0’ | ‘1’ | ‘2’ | ‘3’ | ‘4’ | ‘5’ | ‘6’ | ‘7’ | ‘8’ | ‘9’

NameSeg := [NameChar NameChar NameChar NameChar] |

[NameChar NameChar NameChar ‘_’] |

[NameChar NameChar ‘_’ ‘_’] |

[NameChar ‘_’ ‘_’ ‘_’]

Yes, ‘____’ is a legal term.

MultiNamePrefix := MultiNamePrefixOp ByteConst

For example, MultiNamePrefix(x23) => x11 x23 and refers to a x23 == 35 decimal segment name (unreasonably long.)

NameTail := NameSeg |

[MultiNamePrefix(segmentcount) NameSegsegmentcount]  |

[DualNamePrefix NameSeg NameSeg]

ParentNameHead := [ParentNameHead ParentNamePrefix] | [ParentNamePrefix]

DualNamePrefix NameSeg NameSeg has a smaller encoding than the equivalent utterance MultiNamePrefix(2) NameSeg NameSeg.

‘\_____’ refers to the ROOT, it is used mostly in Scope operators.

SuperName := Name | Arg | Local  | ReturnArg

OpCode := MachineCode | FunctionCode | SuperName

ShortConstants := ZeroOp | OneOp | OnesOp

Revision := RevisionOp

Debug := DebugOp

Local := Local0 | Local1 | Local2 | Local3 | Local4 | Local5 | Local6 | Local7

Arg := Arg0 | Arg1 | Arg2 | Arg3 | Arg4 | Arg5 | Arg6

DefAlias := DefAliasOp Name Name

NamedObjectList := [NamedObjectList  NamedObject] | NamedObject

NamedObject :=  DefMethod | DefSync | DefRegion | DefField | DefDevice |

DefProcessor | DefPowerResOp | DefThermalOp |

DefName | DefNameInst

DefName := DefNameOp  Name  Data

DefNameInst := DefNameOp  Name  Data

Scope := ScopeOp Name  NamedObjectList

DefByte := DefByteOp ByteConst

DefWord := DefWordOp WordConst

DefDWord := DefDWordOp DWordConst

DefString := DefStringOp Asciib NullChar

Asciib := [Asciib AsciiChar] | AsciiChar

DefBuffer := DefBufferOp ByteConst(buffsize) ByteConstbuffsize

Data := [DataTerm Data] | DataTerm

DataTerm := DataItem | DefAnonPkg

DataItem := DefByte | DefWord | DefDWord | DefString | DefBuffer

DefAnonPkg := DefAnonPkgOp PkgLength(length(Data)) Data

For example, an anonymous package consisting of a Byte and a Dword would be encoded as:

DefAnonPkgOp PkgLength(7) DefByteOp ByteConst() DefDWordOp DWordConst()

0x12 x07 x0a x?? x0c x?? x?? x?? x??

CodePkg := DefCodePkgOp PkgLength CodeList

CodeList := [CodeList OpCode] | OpCode

DefMethod := DefMethodOp Name MethodFlags PkgLength CodeList

MethodFlags |= 0-2: ArgCount

3-7: reserved

ArgCount := 0-7

DefSync := DefSyncOp Name SyncFlags

SyncFlags |= 0-3 SyncLevel => x0 - xf

4-7 SyncType

SyncType:4 := Mutex | PulseEvent

Mutex => 0

PulseEvent => 1

DefRegion := DefRegionOp Name RegionSpace

RegionAddress  RegionLength

RegionSpace := SystemMemory 		=> 0 |

SystemPort 		=> 1 |

PciConfig		=> 2 |

EmbeddedControl  	=> 3

Offset := DWordConst

Length := DWordConst

DefField := DefFieldOp Name FieldFlag PkgLength DefFieldList

DefFieldList := [DefFieldList [Name | NullChar] ByteConst] |

[Name | NullChar] ByteConst

FieldFlag |= 0-0: LockRule

1-1: ParentScope

2-3: reserved

4-5: AccessType

6-7: reserved

LockRule:1 := Lock | NoLock

Lock     =>  1			// protect with global lock (all parent obj acc)

NoLock =>  0

ParentScope:1 := Parent | NotParent

Parent       => 1			// field is a name space peer of parent object

NotParent => 0

AccessType := AnyAcc | ByteAcc | WordAcc | DWordAcc

AnyAcc       => 0

ByteAcc      => 1

WordAcc    => 2

DWordAcc => 3

DefDevicePkg := DefDevicePkgOp Name ByteConst(0) PkgLength NamedObjectList

DefProcessorPkg := DefProcessorPkgOp Name

ProcFlags ApicID WordConst(0)

PBlk PBlkLen

PkgLength

NamedObjectList

ProcFlags |= 0-0: BootProcessor => 0 or 1

1-7: reserved

ApicID := ByteConst

PBlk := DWordConst

PBlkLen :=ByteConst

???DefPowerResourcePkg :=

DefPowerResourcePkgOp Name SysLevel Level PkgLength NamedObjectList

SysLevel := Name

Level := Name 			// this makes no sense

???DefThermalPkg := DefThermalOp

ReservedObject := ReservedOp PkgLength(length) ByteConstlength

MachineCode := Noop | Store | StoreConstant | RefStore | Add | Concat | Subtract | Increment | Decrement | Multiply | Divide | ShiftRight | ShiftLeft | SetleftBit | SetRightBit | BitAnd | BitNAnd | BitOr | BitNor | BitXor | CondRefStore | RefIndex | RefMatch | Field | DwordField | WordField | ByteField | BitField | Type | LAnd | LOr | LEqual | LNotEqual | LGreater | LGreaterEqual | LLess | LLessEqual | If | IfElse | While |  Return

Noop := NoopCode

Source := OpCode

Source1 := OpCode

Source2 := OpCode

Count := OpCode

BitNum := SuperName

Result := SuperName

Destination := SuperName

Index := OpCode

Store := StoreOp Source Destination

RefStore := RefStoreOp SuperName SuperName

Add := AddOp Addend1 Addend2 Result	                              // add add1 add2 sum

Addend1 := OpCode

Addend2 := OpCode

Concat := ConcatOp Source1 Source2 Destination

Subtract := Subtract Op  Minuend Subtraend Difference	// result = Minuend-Subtraend

Minuend := OpCode

Subtraend := OpCode

Difference := SuperName

Increment := IncrementOp SuperName

Decrement := DecrementOp SuperName

Multiply := MultiplyOp Multiplican Multiplier Product

Multiplican := Opcode

Multiplier := opcode

Product := SuperName

Divide := DivideOp  Dividend Divisor Remainder Quotient

Dividend := Opcode

Divisor := Opcode

Remainder := SuperName

Quotient := SuperName

ShiftRight : = ShiftRightOp Source Count Result

ShiftLeft := ShiftLeftOp Source Count Result

SetLeftBit : = SetLeftBitOp Source BitNum

SetRightBit := SetRightBitOp Source BitNum

BitAnd := BitAndOp Source1 Source2 Result

BitNAnd := BitNAndOp Source1 Source2 Result

BitOr := BitOrOp Source1 Source2 Result

BitNor := BitNorOp Source1 Source2 Result

BitXor := BitXOrOp Source1 Source2 Result

CondRefStore := CondRefStoreOp Source Destination

RefIndex := RefIndexOp Source Index Destination

RefMatch := RefMatchOp Source Index Destination

Field := FieldOp Source Offset NoBits Index Destination

Offset := Opcode

NoBits := Opcode

DwordField := DwordFieldOp Source Index Destination

WordField := WordFieldOp Source Index Destination

ByteField := ByteFieldOp Source Index Destination

BitField := BitFieldOp Source Index Destination

Type := TypeOp SuperName

LAnd := LAndOp Source1 Source2

LOr := LOrOp Source1 Source2

LEqual := LEqualOp Source1 Source2

LNotEqual := LNotEqualOp Source1 Source2

LGreater := LgreaterOp Source1 Source2

LGreaterEqual := LgreaterEqualOp Source1 Source2

LLess := LLessOp Source1 Source2

LLessEqal := LLessEqualOp Source1 Source2

Predicate := OpCode

TrueCode := [OpCode | CodePkg]

FalseCode := [OpCode | CodePkg]

If := IfOp Predicate TrueCode

IfElse  := IfElseOp Predicate TrueCode FalseCode

While := WhileCode Predicate TrueCode

Return := ReturnOp ReturnValue

ReturnValue := Opcode

FunctionCode := Load | Delay | Sleep | AcquireWait | ReleasePulse | Call | ChainedCall | FromBCD | ToBCD | Fatal

Load := LoadOp TBD

Delay := DelayOp Microseconds

Microseconds := ByteConst

Sleep := SleepCode Milliseconds

Millisecons := WordConst

AcquireWait := AcquireOp SuperName

ReleasePulse := SignalOp SuperName

Call := CallOp Count Method MethodArgcount

Count := ByteConst

Method := SuperName

MethodArg := OpCode

ChainedCall := ChainedCallOp Count Node MethodArgcount

Count := ByteConst

Node := SuperName

MethodArg := OpCode

FromBCD := FromBCDOp BcdValue Destination

BcdValue := OpCode

ToBCD := ToBCDOp Value Destination

Value := OpCode

Fatal := FatalOp FType FCode FArg

FType := ByteConst

FCode := DwordConst

FArg := OpCode

# ASL: ACPI Control Method Source Language

ASL is principally a declarative language.  Most of it’s source statements can be translated into AML with little more that macro processing.  References to names from the AML grammar have the same meaning in ASL as they have in AML.

#### Grammar:

If a term is not defined in ASL, but is defined in AML, use the AML definition.

ASL statements delcare objects.  Each object has three parts, two of which may be null.

Object := ObjectType FixedList VariableList

FixedList refers to list, of known length, which supplies data that all instances of a given ObjectType must have.  It is written as ( a , b , c , … ) where the number of arguments depends on the specific ObjectType, and some elements may be nested objects, that is (a, b, (q, r, s, t), d).  Arguments to a FixedList may have default values, in which case they may be skipped.  Thus, (a,,c) will cause the default value for the second argument to be used..   Some ObjectTypes may have a null FixedList, which is simply omitted.

Trailing arguments may be left out of a fixed list, in which case the default value is used.

VariableList refers to a list, NOT of predetermined length, of child objects that help define the parent.  It is written as { x, y, z, aa, bb, cc } where once again any argument may be a nested object.  ObjectType determines what terms are legal elements of the VariableList.  Some ObjectTypes may have a null variable list, which is simply omitted.

Multiple blanks are the same as one.  Blank, (, ), ‘,’ and newline are all token separators.

// marks a comment, which continues from the // to the end of the line.

! marks a comments, which continues from the ! to the end of the line.

“” surround an ascii string.

Case is not significant.

Numeric constants may be written in 2 ways, ordinary decimal, or hex, using the notation xhh

nothing

An empty item.  For example { nothing } is equivalent to {}

namepath := nameseg | [namepath ‘.’ nameseg]

The following part of a name.  Trailing _s needed by AML are filled in the by ASL compiler, so ABC.DEF is the same as ABC_.DEF_.   (And both are legal ASL syntax.)

namestr :=  [rootpath namepath] | [prefixpath namepath] | namepath

rootpath := ‘\’

‘\’ occurs only as the first character of a namestr and anchors the name at the root of the ACPI name space.  It is always followed by a nameseg.  (i.e. there is no accessible object named ‘\’, but ‘\’ is a legal argument to scope operators and the like, it is encoded as ‘\____’.)

prefixpath := [‘^’ prefixpath] | ‘^’

‘^’ occurs only at the beginning of  a namestr, though there may be more than one.  It anchors the name in the parent object’s level in the name tree.

See name encoding example in the examples section.

NamedList := [NamedList NamedTerm] | NamedTerm

NamedTerm :=

Alias |

Name |

Field |

Mutex |

Event |

Device |

OperationRegion |

PowerResource |

Processor |

Method

SuperName := namestr | Local | Arg | ReturnArg

Declarators:

Num(integervalue)

integervalue := [ByteConst | WordConst | DWordConst]

Declare an anonymous DWORD variable, and initialize with a byte, word, or dword value as is appropriate.

String(“string_value”)

string_value := Asciib

Declare an anonymous string, of size equal to string_value and with initial value string_value.   The default value is the null string, which has 0 bytes available for storage of other values.

Buffer (Count)  { initializer }

Count := ByteConst

initializer := “string” | bytelist

bytelist := ByteConst bytelist | ByteConst

Declares a Buffer, of size Count and initial value initializer.  If count is not specified, it defaults to the sise of initializer.  It is an error for count to be too small to hold the value specified by the initializer.

Examples (all three generate the same datum in the ACPI space, though they have different AML encodings):

buffer(10) { “P00.00A” }

buffer(10) { x50 x30 x30 x2e x30 x30 x41 }

buffer(10) { x50 x30 x30 x2e x30 x30 x41 x00 x00 x00 }

buffer { x50 x30 x30 x2e x30 x30 x41 x00 x00 x00 }

Alias(sourcename, aliasname)

sourcename := namestr

aliasname := namestr

Creates a new name, aliasname, which refers to and acts exactly like sourcename.

Example:

alias(\SUS.SET.EVEN, SSE)

Name(objectname, targetobject)

objectname := namestr

targetobject := Num | String | Buffer | Package

Attaches targetobject to objectname in the Global ACPI name space.

Example:

name(\PTTX,			// Port to Port Translate Table

package { package { x43, x59 }, package { x90, xff }} )

which could also be written as:

name(\PTTX, { { x43, x59 }, { x90, xff }})

Scope(scopebasename) { scopelist }

scopebasename := namestr

scopelist := [ scopelist NamedTerm] | NamedTerm | nothing

Used to give a base scope to a collection of objects.  All object names defined within the scope act relative to scopebasename.

scopebasename does NOT have to be below the surronding scope.

Examples (see also the name entry in the example section)

scope(\PCI0) {

name(X, NUM(3)),

scope(\) {

method(_RQ_) { return(0) }

},

name(^Y, NUM(4))

}

Defines:

\PCI0.X

\_RQ

\Y

Package (count) { nothing | PackageList }

count := Num

PackageList := [PackageList  [Num | String | Buffer | Package ]] |

[Num | String | Buffer | Package]

Declares an anonymous aggregation of data items.  If count is absent, it is set to match the number of elements in PackageList.  If count is present and greater than the number of elements, the default entry undefined will be used to fill out the package.  It is an error for count to be less than the number of elements in PackageList.  Evaluating an undefined element will yield an error, but they may be assigned values to make them defined.

Examples:

These 2 are the same:

package() { string(“com1”), num(x3f8) }

package { string(“com1”), num(x3f8) }

Package {

Num(3),

Num(9),

String(“ACPI 1.0 COMPLIANT”),

Package {

string(“CheckSum=>”),

Package  {

num(7),

num(9)

}

},

num()

}

The next one amounts to a two dimensional array:

Package {

Package { num(11), num(12), num (13) },

Package { num (21), num(22), num (23) }

}

The next one is legal, but of no apparent use:

Package {}

This one allocates space for 10 things to be defined later (see Name and Index operator):

package (10) {}

CodePackage() { nothing | CodeList }

CodeList := [CodeList   Statement ] |

Statement  |

nothing

Method(methodname, argcount) { CodeList  }

methodname := namestr

argcount := 0-7

Declares a procedure which may be invoked to perform computation.

Mutex(mutexname, level)

mutexname := namestr

level := 0-15

Creates a mutex named mutexname, with level from 0 to 15 specified by level.

See also: Event

Event(eventname)

eventname := namestr

Creates a pulsed event object with name eventname.  See also Mutex

OperationRegion(regionname, RegionSpace, Offset, Length)

regionname := namestr

RegionSpace := SystemMemory | SystemIO | PCI_Config | EmbeddedControl

Offset := DWordConstant

Length := DWordConstant

Defines an operational region.  Offset is the offset within the selected RegionSpace that the region starts at (byte granular) while Length is simply the lengh of the region in bytes.

Field (basename, AccessWidth, LockRule, AccessType ) { FieldList }

basename := namestr

AccessWidth := AccessAsByte |

AccessAsWord |

AccessAsDWord |

AccessAsAny	default is AccessAsAny

LockRule := UseGlobalLock | NoLock, defaults to NoLock

AccessType := Preserved | WriteZeros | WriteOnes, defaults to Perserved

FieldList := [FieldList ‘,’ ‘(‘[Name | nothing ] ByteConst]’)’ |

‘(‘ [Name | nothing] ByteConst ‘)’

Creates a set of fields.  The fields are parts of the object named by basename, but their names appear in the scope surrounding the Field statement.

If the field name is nothing the field is “ingnored” or “reserved”.

Example:

scope(\PCI0) {

field(\GIO0.GRPA, AccessAsDWord, UseGlobalLock, Preserve)

{

( , 1),		! 1 bit reserved field, may be used by SMM, etc

(SLT0, 2),	! 4 state power control for SLT0

(SLT1, 2),	! same for SLT1...

(SLT2, 2),

(SLT3, 2),

( , 1),

(WATT, 20),	! used wattage counter

(, 2)

}

}

Device(devicename) { NamedList }

busdevname := namestr

Creates a Device object, which represents either a Bus, or a Device, or any other such entity of use.

Device opens a name scope.

Example:

scope(\) {

Device(PCI0) {

Device(SLT0) {

method(SET, 1) {

. . .

}

method(GET, 1) {

....

return()

}

} ! end of device SLT0

} ! end of device PCI0

} ! end of scope

Would create names:

\

PCI0

SLT0

SET

GET

Processor(processorname, ApicID, PBlk, PBlkLen) { NamedList }

processorname := namestr

ApicID := ByteConstant

PBlk := DWordConstant

PBlkLen := DWordConstant

Declares a processor.   ACPI expects processors to be named \CPU.x where x is 0, 1, 2, etc.  X is a decimal number.

PBlk is the address of the ACPI processor register block for this CPU, while PBlkLen is its length in bytes.

PowerResource(resourcename, systemlevel, resourceorder) { NamedList }

resourcename := namestr

systemlevel := nameseg

resourceorder := Byte Constant

???

??? ThermalResource

Built In Operators and Functions:

Statement :=

Stall |

Sleep |

AcquireWait |

ReleasePulse |

Reset |

Store |

StoreRef |

Add |

Concat |

Increment |

Decrement |

Multiply |

Divide |
Shift |

Set |

BitAnd |

BitNand |

BitOr |

BitNor |

BitXor |

BitNot |

ConditionalStoreRef |

Index |

Match |

Field |

Type |

And |

Or |

Equal |

Not |

Greater |

Less |

GreaterOrEqual |

LessOrEqual |

If |

While |

Return |

Arg |		// same as for AML, evaluating an Arg may be a method invocation

Local |		// same as for AML, evaluationg a Local may be a method invocation

ReturnArg |

Load |

Call |

ChainedCall |

FromBCD |

ToBCD |

Fatal

SuperStatement :=

Statement |

Num() |

String() |

One |

Zero |

Ones

Stall(count)

count := DWordConstant

Delays execution for At Least count microseconds.  Does NOT yield to other ACPI methods.

Sleep(count)

count := DWordConstant

Yields the processor to other ACPI threads for At Least count *milliseconds*

AcquireWait(name)

name := namestr

If name refers to a Mutex, then the Mutex is acquired.  If it refers to an event, then the calling method blocks waiting for the event to be signaled.   See ReleasePulse()

ReleasePulse(name)

name := namestr

If name refers to Mutex, the Mutex is released.  If it refers to an Event, methods waiting on the event will be awakened.

Reset(name)

name := namestr

???

Store(SRC, DST)

SRC := OpCode

DST := SuperName

Copies SRC to DST

see also StoreRef and ConditionalStoreRef

StoreRef

???

Add(adden1, adden2, sum)

adden1 := OpCode

adden2 := OpCode

sum := SuperName

??? need a table of conversion rules, etc, here.

Concat ???

Increment(value)

value := SuperName

Same as Add(value, 1, value)

Decrement(value)

value := SuperName

Same as Add(value, -1, value)

Multiply(Multiplican, Multiplier, Product)

Multiplican := SuperStatement

Multiplier := SuperStatement

Product := SuperName

Divide(Dividend, Divisor, Remainder, Quotient)

Divident := SuperStatement

Divisor := SuperStatement

Remainder := SuperName

Quotient := SuperName

ShiftRight ???

ShiftLeft ???

SetRight ???

SetLeft ???

Bitwise operators:

BitAnd(src1, src2, result)

BitNand(src1, src2, result)

BitOr(src1, src2, result)

BitNor(src1, src2, result)

BitXor(src1, src2, result)

For all 5 cases, src1 and src2  := SuperStatement  and result := SuperName

Each operator computes the expected bitwise operation both stores it in result and returns it.

The compiler provides:

BitNot(src1, result) := BixXor(src1, Ones, result)

ConditionalStoreRef()

Index

Match

Field ??? What does this do ???

Type()

Logical operators:

And(src1, src2) 			// true if src1 == TRUE and src2 == TRUE

Or(src1, src2)			// true if src1 == TRUE or src2 == TRUE

Equal(src1, src2)			// true if src1 == src2

Not(src1)			// logical not of src1

Greater(src1, src2)		// true if src1 > src2

Less(src1, src2)			// true if src1 < src2

In all 6 cases, src1 and src2 := SuperStatement.

Each operator performs the logical (not bitwise) operation on the return value of the statement or statements, and returns 0 for FALSE and 1 for TRUE.

The following forms are macros provided by the compiler:

GreaterOrEqual(src1, src2) :=  Or(Greater(src1, src2), Equal(src1, src2))

LessOrEqual(src1, src2) := Or(Less(src1, src2), Equal(src1, src2))

If  (predicate) { Statement | CodeList }

If  (predicate) { Statement | CodeList } else { Statement | CodeList }

predicate := SuperStatement

If predicate evaluates to 0, it is false, and the Else clause will be executed.  If it is Not 0, then it is considered true, and the Then clause will be executed.

While (predicate) { Statement | CodeList }

So long as predicate is true (see “if”) then execute  Statement | CodeList.

Return

Returns to the invoker of the method.

DefinitionBlock (…) { DefList  }

The unit of data and code that the OS will load (as part of the Differentiated Definition Block or as part of an addition Definition Block) which describes either the base system or some large extension (such as a docking station.)  The entire DefinitionBlock will be loaded and compiled by the OS as a single unit, and may be unloaded by the OS as a single unit.

???  KEN - fill in the fixed argument list.

#### Predefined Objects:

These objects are predefined by ACPI.

Zero

Evaluates to numeric value 0, writes to it are ignored.

Same as constant 0.

One

Evaluates to numeric value 1, writes to it are ignored.

Same as constant 1.

Ones

Evalues to xffffffff, writes to it are ignored.

Same as constant xffffffff.

Revision

Evaluates to the version number of the current ACPI interpreter.

Example: if (revision >= 2), do...

if (LOR(Greater(Revision, 2), Equal(Revision,2)) { do whatever }

Debug

Evalutes to nothing.

Writes to this object are printed to the kernel debugger, when it’s active, otherwise they are ignored.

# AML and ASL Examples:

### Examples:

1.	Package  () {

Byte(3),

DWord(9),

String(“ACPI 1.0 COMPLIANT”),

Package (){

string(“CheckSum=>”),

Package (){

byte(7),

byte(9)

}

},

byte()

}

would generate the following AML:

DefAnonPkgOp

PkgLength(29+20)

DefByteOp x03 //27

DefDWordOp x00 x00 x00 x09

DefStringOp ‘A’ ‘C’ ‘P’ ‘I’ ‘ ‘ ‘1’ ‘.’ ‘0’ ‘ ‘ ‘C’ ‘O’ ‘M’ ‘P’ ‘L’ ‘I’ ‘A’ ‘N’ ‘T’

NullChar

DefAnonPkgOp //length=20

PkgLength(12+6)

DefString ‘C’ ‘h’ ‘e’ ‘c’ ‘k’ ‘S’ ‘u’ ‘m’ ‘=’ ‘>’ NullChar  //length=12

DefAnonPkgOp  // length() == 6

PkgLength(4)

DefByte x07

DefByte 0x9

DefByteOp x00

which would be represented with the following byte stream:

x12 x31 x0a x03 x0c x00 x00 x00 x09 x0d x41 x43 x50 x49

pkg pln byt      dd                 str  A   C   P   I

x20 x31 x2e x30 x20 x43 x4f x4d x50 x4c x49 x41 x4e

1     .      0       C   O   M   P   L   I   A   N

x54 x00 x12 x12  x0d x43 x68 x65 x63 x6b x53 x75 x6d x3d

T      pkg pln  str  C   h   e   c    k   S  u   m    =

x3e x00 x0a x06  x0a x07 x0a x09 x0a x00

•	pkg pln  byt     byt     byt

=======================================================

#### Names:

Assume:

\

S0

MEM

SET

GET

S1

MEM

SET

GET

CPU

SET

GET

And that LoadOp(‘\S0.CPU.SET’, defblock) occurs, which will create \S0.CPU.SET and load

the block block using it as a root.  Then if the block contains:

STP1

^GET

^^PCI0

^^PCI0.SBS

\S2

\S2.ISA.COM1

^^^S3

^^^S2.MEM

^^^S2.MEM.SET

scope(\S0.MEM.SET.STP1) {

XYZ

^ABC

^ABC.DEF

}

They will be encoded as:

'STP1'

ParentNamePrefix 'GET_'

ParentNamePrefix ParentNamePrefix 'PCI0'

ParentNamePrefix ParentNamePrefix DualNamePrefix 'PCI0' 'SBS_'

RootPrefix 'S2__'

RootPrefix MultiNamePrefix 3 'S2__' 'ISA_' 'COM1'

ParentNamePrefix ParentNamePrefix ParentNamePrefix 'S3__'

ParentNamePrefix ParentNamePrefix ParentNamePrefix DualNamePrefix 'S2__' 'MEM_'

ParentNamePrefix ParentNamePrefix ParentNamePrefix MultiNamePrefix3 'S2__' 'MEM_' 'SET_'

And Will yield

\

S0

MEM

SET

GET

CPU

SET

STP1

XYZ

ABC

DEF

GET

PCI0

SBS

S1

MEM

SET

GET

CPU

SET

GET

S2

ISA

COM1

MEM

SET